INFOMATEK # Volume 9 Nomor 1 Maret 2007

# Kinetics of an Azo Dye Decolourization using UV/H2O2 Technique

Gatut Sudarjanto *)

Jurusan Teknik Lingkungan

Fakultas Teknik – Universitas Pasundan

Abstract: Advanced oxidation process using UV/H2O2 was used to remove CI Reactive Red 195A from textile wastewater. UV radiation source was a low-pressure mercury arc lamp, 60W emitting at 253.7nm. In this work, the effects of initial hydrogen peroxide dosage, dye concentration, pH and temperature were examined to study the kinetics of dye decolourisation through UV/H2O2 process. The aim is to find the optimum treatment conditions for practical applications of dye removal from wastewater and to obtain information relevant for the possible scale-up of the degradation process. In general, the result demonstrates that the decolourization reaction was found to follow first order kinetics with respect to the dye concentration.

Keywords: Azo-dye, Decolourization, Kinetics, Textile, UV/H2O2

## I.  Introduction

The strong colour and high concentrations of organic compounds contained in the textile effluents are the main environmental problems as they are often only partially removed in a conventional wastewater treatment plant, Lucas [1]. Unnatural colours, such as red and violet, commonly raise complaints, Carliell [2] while the public tends to accept blue, green, and brown colours somewhat more, Eston [3], Oneill [4]. The presence of the dye compounds and their by-products on the receiving water bodies does not only inhibit sunlight penetration into the water, which is needed for the photosynthesis process, but also has direct harmful effects on the aquatic microbial populations. Even if the coloured effluent does not have a direct effect on the receiving water bodies, the discharge is of concern when treated water is later utilized for drinking water, Loyd [5] or agricultural proposes, Gravecet [6].

*) Dosen Jurusan Teknik Linkungan FT-Unpas

Currently, colour removal from textile effluent is becoming a major concern, as legislation is becoming stricter. Consequently, the textile industry is forced to treat its coloured effluents more thoroughly by installing advanced treatment facilities to fulfil higher effluent standards based on the legislation and to recycle its water to reduce the increasing costs of water. Hence establishing advanced technologies for removing or degrading synthetic or natural dyes from wastewater has become an important issue.

The advanced oxidation processes (AOPs) that focus on the production of highly reactive hydroxyl radicals (OH), are known as a highly effective unit operation for removing colour and substantial organic materials from textile effluent,Forgacs [7],Neamtu [8],Komprivanac [9]. In the present work, the UV/H2O2 treatment method was utilized to study the kinetics of synthetic dye decolourization in order to obtain information relevant for the chemical reaction and possible scale-up of the degradation process. In the kinetic study of photochemical reactions, the reaction rate can be determined by observing either the disappearance of one reactant or the formation of the product. In this study, reaction rate is defined as the rate of change in the dye concentration (one reactant) with time of irradiation.

## II.  Materials and Methods

### Reagents

The dye chosen for the study was Colour Index Reactive Azo Red 195A (a vinylsulphone mono azo dye) that was obtained from the Logan Textile Company, Queensland, Australia. The dye was used in the experiment without further purification. Analytical grade H2O2 solution (30% w/w) was obtained from Merck (Germany). Analytical grade catalase solution (C-100) from cow liver (Sigma Aldrich, Germany) in K2HPO4 buffer solution (0.05 M, pH 7) was used to remove all residual H2O2 from the samples. H2SO4 and NaOH were used to adjust the pH to the desired value. All the other chemicals were at reagent grade quality and solutions were prepared with MilliQ® water

### Photoreactor

The UV reactor is an Ultraviolet Technology of Australasia model LC-20 (UVTA, Australia) with the total volume of 585ml. The UV light irradiates the water as it flows through the activated fluoropolymer (AFP840) tubing within the unit. The AFP840 teflon type tube is transparent, non-wettable, and chemically inert. The UV reactor is fitted with a fixed low-pressure mercury lamp, 60W emitting at 253.7nm, achieving a UV dosage of more than 40 mWs/cm2. The UV lamp and the AFP tube are covered by a metal casing. The UV intensity was kept constant during all experiments. The lamp was warmed up for a few minutes before the experiment was started to ensure a constant UV flux and eliminate temperature variations.

### Kinetic Rate Constants

In the kinetic study, the decolourization rate can be calculated from the slope of the dye concentration against time for a particular concentration at a specific time. As has been found in previous studies, Galoindo [10], El-Dein [11], Libra[12] the dye degradation is usually a first order reaction with respect to the dye concentration. This is also confirmed in the present study as described in the results section. The main equations characterising a first order reaction can be described as,

| −dCtdt=kobsCt | Eq. 1 |
| --- | --- |
| lnCtCo=−kobst | Eq. 2 |
| t12=0.693kobs | Eq. 3 |

where Ct is the concentration of the dye at a certain irradiation time (t), and kobs is the (observed) first order rate constant. The kinetic rate constants kobs were obtained from the linear regression of the ln(Ct/Co) vs. t data (Eq. 1-2). These reactions are often also described by their half-life reaction time (as described on Eq. 3), symbolized by t1/2 (min). In theory, the half-life reaction time for first order kinetics is independent of the initial concentration of the reactant.

### Analytical Methods

In the present study, the hydrolysed dye stock solution was prepared according to the common dyebath application conditions recommended by the textile company. Five grams of powdered dye and 40 g of NaOH pellets were dissolved in approximately 500 ml of MilliQ® water and the pH adjusted to 12 with 1 M H2SO4 solution. The solution was stirred for 1 hour at 80ºC. After cooling to room temperature, the solution was neutralized to pH 7 with H2SO4 and diluted to 1 litre with MilliQ® water. The dye was used as the sole organic compound in these tests with a concentration of about 100 mg dye/L, corresponding to 90 to 100 mg sCOD/L.

A spectrophotometric method, Apha [13] using Photometer SQ 118 (Merck, Germany) with a wavelength of 550nm was used to measure the dye concentration of filtered samples from the influent and effluent of the photoreactor.

The H2O2 solution was diluted and added directly to the influent just before entering the photoreactor. The initial and residual concentrations of H2O2 were determined by Reflectoquant Peroxide test strips (Merck, Germany).

Chemical oxygen demand (COD) was measured according to the Standard Methods (APHA, 1998) using a Thermoreactor TR 300 (Merck, Germany) and absorbance measurements were made using the Photometer SQ 118 (Merck, Germany). Residual H2O2 was eliminated using catalase solution before the sCOD measurement to avoid incorrect results.

500l of catalase (aqueous suspension) was added to 100ml of 0.05M potassium phosphate buffer, pH 7, Koh [14]. 100μl of this catalase-K2HPO4 buffer solution was then added to each sample after neutralization with NaOH or HCl to remove any remaining H2O2 from the samples as this could otherwise interfere with the COD measurements.

Temperature and pH levels were measured using a Metrohm 744 pH sensor. Samples were taken every 2.5 minutes. All samples were filtered using a syringe filter (0.22 μm Millex® GP, Bedford, USA) and diluted when necessary before analysis. Simulated wastewater was prepared freshly before each irradiation experiment. Control experiments were carried out under UV irradiation without H2O2 in the solutions and with H2O2 but without UV irradiation.

## III.  Results and Discussion

### Effect of Initial H2O2 Dosage

To study the effect of H2O2 concentration on the decolourization rate, 7 different concentrations of H2O2 (100, 200, 300, 600, 900, 1500, and 3000mg/l) were added to the dye solution. Figure 1 shows the decolourization of RR195A as a function of UV irradiation time for various initial H2O2 dosages.

Figure 1

Effect of different initial H2O2 dosage on the decolourization of RR195A using the UV/H2O2 method. Initial dye concentration=100mg/L, pH 8, temperature=30oC.

The logarithmic representation of RR195A removal against irradiation time is shown in 2. The observed first order kinetic rate constants for colour removal for different initial hydrogen peroxide concentrations are summarized in  and plotted in 3. The linear regression lines fit the data very well in this logarithmic plot, which is also supported by the high correlation coefficients (R2 values in ) for all concentrations studied. This clearly indicates that the reaction seems to progress according to first-order kinetics as mentioned above (especially from 5 to 12.5 minutes of irradiation time).

Figure 2

Logarithmic representation of RR195A decolourization as a function of different initial H2O2

dosages. Initial dye concentration=100mg/L, pH 8, temperature=30oC.

Table 1

Kinetic rate constants of dye removal using the UV/H2O2 method

(as a function of different initial H2O2 dosages)

| H2O2 | Equations | kobs (min-1) | R2 | t1/2 (min) |
| --- | --- | --- | --- | --- |
| 100 | Ct/Co=e-0.0914[H2O2](t) | 0.0914 | 0.9831 | 7.58 |
| 200 | Ct/Co=e-0.1485[H2O2](t) | 0.1485 | 0.9595 | 4.67 |
| 300 | Ct/Co=e-0.2540[H2O2](t) | 0.2540 | 0.9920 | 2.73 |
| 600 | Ct/Co=e-0.3214[H2O2](t) | 0.3214 | 0.9932 | 2.16 |
| 900 | Ct/Co=e-0.4175[H2O2](t) | 0.4175 | 0.9836 | 1.66 |
| 1500 | Ct/Co=e-0.4459[H2O2](t) | 0.4459 | 0.9877 | 1.55 |
| 3000 | Ct/Co=e-0.4566[H2O2](t) | 0.4566 | 0.9818 | 1.52 |

Figure 3

Effect of different initial H2O2 concentrations on the kinetic rate constants of RR195A decolourization.

Initial dye concentration=100mg/L, pH 8, temperature=30oC.

The highest rate of colour removal was obtained at an initial H2O2 concentration of 3000 mg/L (kobs = 0.4566 min-1, t1/2 = 1.52 min), while the lowest removal rate occurred at a peroxide concentration of 100 mg/L (kobs = 0.0914 min-1, t1/2 = 7.58 min). However, as is clearly shown in Figure 3, the removal rate is only increasing rapidly in the lower H2O2 concentration range (shown in more detail in ), while it does no longer increase substantially above an initial concentration of 900 mg/L H2O2. This is likely due to the increasing side reactions of H2O2 with hydroxyl radicals or other reactive species as discussed above. This leads to a drastically reduced efficiency of the H2O2 utilisation for the dye degradation reaction.

On the other hand, in the lower H2O2 concentration range (up to 300 mg H2O2/L), the observed reaction rate seems to be directly proportional to the initial peroxide concentration (Figure ). This indicates that the reaction at least in this concentration range is not only proportional to the dye concentration (hence the first order reaction rate with respect to the dye concentration) but also to the initial peroxide concentration. It has to be noted though, that peroxide is also a reactant and therefore being consumed in the reaction, but this does not appear to lead to a further reduced reaction rate with respect to the actual peroxide concentration in the reactor. If this was the case, then the overall reaction would be expected to progress according to second order kinetics as both reactants (dye and H2O2) are being reduced in the process. Instead, the effect of the initial H2O2 concentration seems to be mainly related to the rate at which hydroxyl radicals are formed in the system. The UV-induced production could be expected to be faster at higher H2O2 concentrations in the liquid. This is leading to a higher hydroxyl radical formation rate and therefore increased reaction rates for the dye degradation. While this proposed mechanism could not be confirmed in the present system, it is consistent with investigations reported in the literature,Legrini [15],Siminiceanu [16], as well as with the data observed in this study.

Figure 4

Effect of different initial H2O2 concentrations on the first order rate constants of RR195A decolourization. Initial dye concentration= 100mg/L, pH 8, temperature=30oC.

### Effect of Initial pH

The effect of different pH values (3, 6, 7, 8, 9, 10, 11 and 12) on RR195A decolourization using the UV/H2O2 system was examined. It is known that the pH of the solution will determine the efficiency of the UV/H2O2 treatment process. Figure 5 shows the decolourization rate of RR195A at various pH levels.

Figure 5

Effect of different pH levels on the decolourization of RR195A using the UV/H2O2 method. Initial dye=100mg/L, H2O2=300mg/L, temp.=30oC.

From 6 we can see the logarithmic diagram of dye removal versus irradiation time as a function of the (initial) pH.  shows the kinetic data of RR195A decolourization. The highest decolourization rate was obtained at the initial pH of 3 (kobs = 0.275 min-1, t1/2 = 2.52 min), while the lowest rate at pH 12 was nearly four times slower (kobs = 0.0733 min-1, t1/2 = 9.45 min).

Figure 6

Logarithmic representation of RR195A decolourization as a function of different initial pH. Initial dye concentration =100mg/L, H2O2=300 mg/L, temperature=30oC.

Table 2

Kinetic rate constants of dye removal using the UV/H2O2 method (as a function of different initial pH)

| pH | Equations | kobs (min-1) | R2 | t1/2 (min) |
| --- | --- | --- | --- | --- |
| 3 | Ct/Co=e-0.2750(pH)(t) | 0.2750 | 0.9932 | 2.52 |
| 6 | Ct/Co=e-0.2495(pH)(t) | 0.2495 | 0.9946 | 2.78 |
| 7 | Ct/Co=e-0.2403(pH)(t) | 0.2403 | 0.9914 | 2.88 |
| 8 | Ct/Co=e-0.2500(pH)(t) | 0.2500 | 0.9981 | 2.77 |
| 9 | Ct/Co=e-0.2039(pH)(t) | 0.2039 | 0.9989 | 3.40 |
| 10 | Ct/Co=e-0.1682(pH)(t) | 0.1682 | 0.9986 | 4.12 |
| 12 | Ct/Co=e-0.0733(pH)(t) | 0.0733 | 0.9189 | 9.45 |

Considering the pH effect on the degradation rate (), it is clear that there is a minor impact up to pH 8, while above this level the effect is very significant. 8 demonstrates the initial pH in the range of 8-12 seems to have a direct proportional effect on the first order rate constants of dye degradation using the UV/H2O2 system. The cause for this apparent linear relationship is not clear, particularly also since the pH measure itself is a logarithmic function of the H+ concentration in the solution. Although this can not be easily explained, it is of significant practical relevance as the degradation rate is reduced by about 17 % for every pH unit above pH 8.

Figure 7

Effect of different initial pH on the kinetic rate constants of RR195A decolourization. Initial dye concentration=100mg/L, H2O2=300 mg/L, pH 8, temperature=30oC.

Figure 8

Effect of different initial pH in the range of 8-12 on the first order rate constants of RR195A decolourization. Initial dye concentration=100mg/L, H2O2=300 mg/L, temperature=30oC.

### Dye Concentration Variations

To investigate the effect of RR195A decolourization using the UV/H2O2 method, 5 different initial dye concentrations were tested. The results are shown in Figure  for a H2O2 concentration of 300 mg/L and a pH = 8.

As can be seen in Figure 10, the rate constants dropped off with increasing dye concentrations, although the initial concentration of H2O2 was the same. The semi-log charts of Ct/Co against time (for the first 15 minutes irradiation) were linear confirming the first order reaction kinetics of the dye removal rate with respect to the dye concentration. Interestingly, the reaction rates (particularly for the lower dye concentrations) seemed to increase slightly when the Ct/Co ratio dropped below 0.2. This may be caused by an increasing amount of UV radiation being absorbed by H2O2 after only a small fraction of colour was remaining. The effect of different initial dye concentration on the kinetic rate constants of RR195A decolourization is shown in Figure 11.

Figure 9

Effect of different initial dye concentration on the decolourization of RR195A using the UV/H2O2 method. Initial H2O2 concentration=300mg/L, pH=8, temperature=30oC.

Figure 10

Logarithmic representation of the RR195A decolourization as a function of different initial dye concentration. Initial H2O2 concentration= 300mg/L, pH=8, temperature= 30oC.

shows the rate constants and their half-life times for RR195A decolourization with different initial dye concentrations. The highest decolourization rate was achieved at the initial dye concentration of 50 mg/L (kobs = 0.1740 min-1, t1/2 = 3.98 min), while the lowest removal rate occurred at dye concentration of 453 mg/L (kobs = 0.0053 min-1, t1/2 = 130.75 min). It is quit obvious that the initial dye concentration does have a direct effect on the overall reaction rate. The rate of the dye degradation slows down with increasing initial concentration, leading to higher half-life periods. This is likely caused by the changing ratio of dye to H2O2, which ranges from 1:6 to 1:0.67 (on mass basis) for these experiments.  This demonstrates that the initial concentration of both reactants is influencing the reaction rate as would be expected since they are both used up in the process.

Table 3

Kinetic rate constants of dye removal using the UV/H2O2 method

(as a function of different dye concentration)

| Dye (mg/L) | Equations | kobs (min-1) | R2 | t1/2 (min) |
| --- | --- | --- | --- | --- |
| 50 | Y=e-0.1740x | 0.1740 | 0.9588 | 3.98 |
| 75 | Y=e-0.0901x | 0.0901 | 0.9846 | 7.69 |
| 100 | Y=e-0.0781x | 0.0781 | 0.9823 | 8.87 |
| 170 | Y=e-0.0289x | 0.0289 | 0.9612 | 23.98 |
| 453 | Y=e-0.0053x | 0.0053 | 0.9067 | 130.75 |

Figure 11

Effect of different initial dye concentration on the kinetic rate constants of RR195A decolourization. Initial H2O2=300 mg/L, pH 8, temperature=30oC.

### Effect of Temperature

Several runs were conducted at temperatures between 15 and 75oC, with H2O2 concentration of 300 mg/L, pH = 8 and initial dye concentration of 100 mg/L to find the effect of temperature on removal rates, The dye removal rate using the UV/H2O2 method increased with increasing temperature of the system as shown in Figure 12.

Figure 12

Effect of different temperatures on the decolourization of RR195A using the UV/H2O2 method. Initial dye concentration=100mg/L, H2O2=300mg/L, pH=8.

shows the logarithmic illustration of RR195A decolourization as a function of different temperatures.  summarized the observed kinetics data, which is plotted in .

Figure 13

Logarithmic representation of the RR195A decolourization as a function of different temperatures. Initial dye concentration=100mg/L, H2O2=300mg/L, pH=8 (=15 C, =30 C, ▲ =55 C, =75C)

Table 4

Kinetic rate constants of dye removal using the UV/H2O2 method (as a function of different temperatures)

| Temperature (oC) | Equations | kobs (min-1) | R2 | t1/2 (min) |
| --- | --- | --- | --- | --- |
| 15 | Ct/Co=e-0.1794T | 0.2084 | 0.9812 | 3.32 |
| 30 | Ct/Co =e-0.2111T | 0.2512 | 0.9952 | 2.76 |
| 55 | Ct/Co =e-0.3135T | 0.3871 | 0.9437 | 1.79 |
| 75 | Ct/Co =e-0.5067T | 0.6942 | 0.8790 | 1.00 |

Figure 14

Effect of different temperatures on the rate constants of RR195A decolourization. Initial dye concentration=100mg/L, H2O2=300 mg/L, pH 8.

Specific reaction-rate constants are dependent on temperature. The temperature dependence of the rate constants can be estimated using the van’t Hoff-Arrhenius relationship,Metcalf [17].

| d(lnk)dT=ERT2 | Eq. 4 |
| --- | --- |

where 	k  = reaction rate constant

T  = temperature, K = 273.15 + oC

E = a constant characteristic of the reaction (e.g. activation energy), J/mol

R = ideal gas constant, 8.314 J/mol•K (1.99 cal/mol•K)

Integration of Eq.  between the limits T1 and T2 gives

| lnk2k1=E(T2−T1)RT1T2=ERT1T2(T2−T1) | Eq. 5 |
| --- | --- |

With k1 measured for a given temperature and with E known, k2 can be estimated using Eq. 5. Alternatively, the activation energy E can be calculated by determining k values at two different temperatures. Because most wastewater treatment operations and processes are carried out at or near the ambient temperature, the quantity E/(RT1T2) in Eq.  may be assumed to be a constant for all practical purposes. If the value of the quantity is designated by C, then Eq. 5 can be rewritten as:

| lnk2k1=C(T2−T1) | Eq. 6 |
| --- | --- |
| k2k1=eC(T2−T1) | Eq. 7 |

Replacing eC in Eq.  7 with a temperature coefficient  yields

| k2k1=θ(T2−T1) | Eq. 8 |
| --- | --- |

or

| k2=k1θ(T2−T1) , | Eq. 9 |
| --- | --- |

which is a commonly used temperature correction function in wastewater treatment processes. Based on the observed results summarized in Table  and using Eq.  we can calculate the  values. It can be seen from Table  5 the values of  at different temperatures.

Table 5

The  values at different temperature operations

| T1 | T2 | K1 | K2 |  (T2-T1) |  |
| --- | --- | --- | --- | --- | --- |
| 15 | 30 | 0.2084 | 0.2512 | 1.20 | 1.01 |
| 15 | 55 | 0.2084 | 0.3871 | 1.54 | 1.01 |
| 15 | 75 | 0.2084 | 0.6942 | 1.79 | 1.01 |
| 30 | 55 | 0.2512 | 0.3871 | 1.86 | 1.03 |
| 30 | 75 | 0.2512 | 0.6942 | 2.76 | 1.02 |
| 55 | 75 | 0.3871 | 0.6942 | 3.33 | 1.06 |

The obtained  values vary from 1.01 to 1.06 at the high temperatures. This indicates that the approximation used above may not be fully consistent at the higher temperatures, or the rate determined at 75°C may not be very accurate, leading to this discrepancy. However, in this study, the  value is considered to be 1.01 as most experiments are conducted in the range of 15 to 30°C. By knowing the value of  and the rate constants at certain temperatures, rate constants at different temperatures can be predicted.

## Conclusion

The present study has shown that the UV/H2O2 method is very effective in decolourizing a synthetic dye (RR195A). From the experiments, at an initial concentration of 100 mg dye/L, excellent colour removal was obtained between 20 and 30 minutes irradiation with an initial H2O2 dosage ranging from 300 to 900 mg H2O2/L. Peroxide concentrations higher than 900 mg H2O2/L did not significantly improve the results due to the scavenging effect of the produced radicals. Higher pH and temperature enhanced the system performance, however for practical application, particularly in the textile industry, a pH of 8 (or lower) and a temperature of 30oC was suggested as a suitable compromise between the likely wastewater characteristics and the achievable colour removal performance. The kinetic analysis showed that the decolourization of RR195A using UV/H202 followed first order kinetics in relation to colour removal. At the same time, the decolourization rate constants were proportional to the initial peroxide concentration.

# V.   References

[1]	Lucas, M. S. & Peres, J. A. (2006). Decolorization of the azo dye Reactive Black 5 by Fenton and photo-Fenton oxidation. Dyes and Pigments, 71(3), 236-244.

[2]	Carliell, C. M., Barclay, S. J., Naidoo, N., Buckley, C. A., Mulholland, D. A. & Senior, E. (1995). Microbial Decolorization of a Reactive Azo-Dye under Anaerobic Conditions. Water Sa, 21(1), 61-69.

[3]	Easton, J. R. (1995). Colour in dyehouse effluent. Oxford: The Alden Press.

[4]	O' Neill, C., Hawkes, F. R., Hawkes, D. L., Lourenco, N. D., Pinheiro, H. M. & Delee, W. (1999). Colour in textile effluents - sources, measurement, discharge consents and simulation: a review. Journal of Chemical Technology and Biotechnology, 74(11), 1009-1018.

[5]	Loyd, C. K., Boardman, G. D. & Michelsen, D. L. (1992). Aerobic, Anaerobic, and Chemical Degradation of a Textile Dye Waste-Water. Abstracts of Papers of the American Chemical Society, 204, 137-ENVR.

[6]	Gravelet-Blondin, L. R., Carliell, C. M., Barclay, S. J. & Buckley, C. A. (1996). Management of water resources in South Africa with respect to the textile industry. 2nd IAWQ Specialised Conference on Pretreatment of Industrial Wastewaters. Athens, Greece.

[7]	Forgacs, E., Chserhati, T. & Oros, G. (2004). Removal of synthetic dyes from wastewaters: a review. Environment International, 30, 953-971.

[8]	Neamtu, M., Yediler, A., Siminiceanu, I., Macoveanu, M. & Kettrup, A. (2004). Decolourization of disperse red 354 azo dye in water by several oxidation process-a comparative study. Dyes and Pigments, 60, 61-68.

[9]	Koprivanac, N., Kusic, H., Vujevic, D., Peternel, I. & Locke, B. R. (2005). Influence of iron on degradation of organic dyes in corona. Journal of Hazardous Materials, B117, 113-119.

[10]	Galindo, C. & Kalt, A. (1999). UV/ H2O2 oxidation of azodyes in aqueous media: evidence of a structure - degradability relationship. Dyes and Pigments, 42, 199-207.

[11]	El-Dein, A. M., Libra, J. A. & Wiesmann, U. (2001). Kinetics of decolorization and mineralization of the azo dye Reactive Black 5 by hydrogen peroxide and UV light. Water Science and Technology, 44(5), 295-301.

[12]	El-Dein, A. M., Libra, J. A. & Wiesmann, U. (2003). Mechanism and kinetic model for the decolorization of the azo dye Reactive Black 5 by hydrogen peroxide and UV radiation. Chemosphere, 52(6), 1069-1077.

[13]	APHA, AWWA & WPCF (1998). Standard Methods for the examination of water and wastewater, 20th Ed. Washington, DC.

[14]	Koh, I. O., Chen-Hamacher, X., Hicke, K. & Thiemann, W. (2004). Leachate treatment by the combination of photochemical oxidation with biological process. Journal of Photochemistry and Photobiology A: Chemistry, 162, 261-271.

[15]	Legrini, O., Oliveros, E. & Braun, A. M. (1993). Photochemical processes for water treatment. Chem. Rev., 93, 671-698.

[16]	Neamtu, M., Siminiceanu, H., Yediler, A. & Kettrup, A. (2002). Kinetics of decolorization and mineralization of reactive azo dyes in aqueous solution by the UV/ H2O2 oxidation. Dyes and Pigments, 53(2), 93-99.

[17]	Metcalf & Eddy (2003). Wastewater Engineering: Treatment and Reuse. New York: McGraw-Hill Co. Inc.